| Customer Name: Mr. Nitin Dhakite Address: 149,,Emerald Anutham Deoguradia Indore (M.P.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sample Name | Drinking Water |  | Test Report No. | FTL/AIPER/25/1812-A |
| Sample Details | 01 x 05 Litre |  | Registration No. | 713 |
| Packing Details | Packed in plastic bottle |  | Issue Date | 14/10/2025 |
| Marking Seal ( if any) | NA |  | Date of Receipt | 25/04/2025 |
| Sample condition on receipt | Satisfactory |  | Testing period | 25/04/2025 to 28/04/2025 |
| Customer ref. | NA |  | Standard Specification | IS 10500:2012 |
| Sampling Details | Sample provided by the customer |  | Brand Name | NA |
| Batch No. | NA |  | Tests requested | As Mentioned below |
| DOM | NA |  | DOE | NA |
| Any Handling Instructions provided : Yes/NO ( if yes ; Short details) |  |  | No |  |
| Any data provided by customer |  |  | NA |  |
| Sample Description |  | Clear colourless liquid without any sediments & suspended particles. |  |  |

TEST RESULT

| Sr. No. | Test Parameters | UoM | Result | Test Method | Specification |
| --- | --- | --- | --- | --- | --- |
| Organoleptic and Physicals Parameters |  |  |  |  |  |
|  | Odour | - | Agreeable | IS : 3025 (Part 5):2018 | Agreeable |
|  | Colour | Hazen | < 2 | IS : 3025 (Part 4):2021 | Max. 5 |
|  | Turbidity | NTU | 0.7 | IS 3025 (part 10) Nephelometric Turbidity Unit (NTU) method :2023 | Max 1 |
|  | pH | - | 7.75 | IS : 3025 (Part 11):2022 | 6.5 – 8.5 |
|  | Total Dissolve solid | mg/L | 969 | IS 3025 (Part 16): 2023 | Max.500 |
| General Parameters |  |  |  |  |  |
|  | Total Hardness (as CaCO3) | mg/L | 461.24 | IS 3025 (part 21) EDTA method:2019 | Max.200 |
|  | Magnesium (as Mg) | mg/L | 51.70 | IS 3025 (Part 46):2019 | Max. 30 |
|  | Copper (as Cu) | mg/L | BLQ (LOQ 0.005) | IS 3025 (Part 2):2019 | Max 0.05 |
|  | Iron (as Fe) | mg/L | 0.013 | IS 3025 (Part 2):2019 | Max 0.3 |
|  | Manganese (as Mn) | mg/L | BLQ (LOQ 0.002) | IS 3025 (Part 2):2019 | Max 0.1 |
|  | Nitrate( as NO3 ) | mg/L | 1.2 | APHA 24th edition. :4500- NO3 -B Ultra Spectrophotometric Screening method, 2023 | Max 45 |
|  | Fluoride(as F) | mg/L | BLQ (LOQ 0.1) | APHA 24 edition:4500-F,D SPADNS Method 2023 | Max 1.0 |
|  | Zinc (as Zn) | mg/L | 0.007 | IS 3025 (Part 2):2019 | Max 5 |
|  | Aluminium (as Al) | mg/L | 0.006 | IS 3025 (Part 2):2019 | Max 0.03 |

Continue…….

| Sample Name | Drinking Water | Test Report No. | FTL/AIPER/25/1812-A |
| --- | --- | --- | --- |
| Sample Details | 01 x 05 Litre | Registration No. | 713 |

| Sr. No. | Test Parameters | UoM | Result | Test Method | Specification |
| --- | --- | --- | --- | --- | --- |
|  | Chloride (as Cl) | mg/L | 198.42 | IS 3025 (Part 32): 1988 (RA 2019) | Max 250 |
|  | Selenium (as Se) | mg/L | 0.004 | IS 3025 (Part 2):2019 | Max 0.01 |
|  | Sulphate (as SO4 ) | mg/L | 27.61 | IS 3025 (part 24)Turbidity method: 2022 | Max 200 |
|  | Alkalinity (as CaCO3 ) | mg/L | 215.78 | IS 3025 (Part 23):2023 | Max 200 |
|  | Calcium (as Ca) | mg/L | 99.40 | IS 3025 (Part 40 /Sec.5) EDTA Titrimetric method: 2024 | Max 75 |
|  | Residual free chlorine | mg/L | BLQ (LOQ 0.01) | IS 3025 ( Part 26):2021 | Max 0.2 |
|  | Mineral oil | mg/L | BLQ (LOQ 0.5) | IS : 3025 (Part 39):2021 | Max.0.5 |
|  | Boron (as B) | mg/L | BLQ (LOQ 0.01) | IS 3025 (Part 2):2019 | Max 0.5 |
|  | Ammonia (as ammonia-N) | mg/L | BLQ (LOQ 0.5) | IS 3025 (part 34 /Sec,1): 2023 | Max.0.5 |
|  | Silver (as Ag) | mg/L | BLQ (LOQ 0.01) | IS 3025 (Part 2):2019 | Max.0.1 |
| Parameters Concerning Toxic Substances |  |  |  |  |  |
|  | Cadmium (as Cd) | mg/L | BLQ (LOQ 0.005) | IS 3025 (Part 2): 2019 | Max 0.003 |
|  | Cyanide as CN | mg/L | BLQ (LOQ 0.02) | APHA 24th edition 4500-CN-E Colourimetric :2023 | Max.0.05 |
|  | Lead (as Pb) | mg/L | BLQ (LOQ 0.005) | IS 3025 (Part 2): 2019 | Max 0.01 |
|  | Chromium (as Cr) | mg/L | BLQ (LOQ 0.005) | IS 3025 (Part 2):2019 | Max.0.05 |
|  | Molybdenum (as Mo) | mg/L | BLQ (LOQ 0.005) | IS 3025 (Part 2):2019 | Max.0.07 |
|  | Mercury (as Hg) | mg/L | BLQ (LOQ 0.001) | IS 3025 (Part 2): 2019 | Max.0.001 |
|  | Total Arsenic (as As) | mg/L | BLQ (LOQ 0.002) | IS 3025 (Part 2): 2019 | Max.0.01 |
|  | Nickel (as Ni) | mg/L | BLQ (LOQ 0.005) | IS 3025 (Part 2):2019 | Max.0.02 |
|  | Polynuclear Aromatic Hydrocarbons (PAH) |  |  |  |  |
| 9.1. | Acenaphthene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.2. | Acenaphtylene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.3. | Anthracene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.4 | Benz(a)anthracene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |

Continue…….

| Sample Name | Drinking Water | Test Report No. | FTL/AIPER/25/1812-A |
| --- | --- | --- | --- |
| Sample Details | 01 x 05 Litre | Registration No. | 713 |

| Sr. No. | Test Parameters | UoM | Result | Test Method | Specification |
| --- | --- | --- | --- | --- | --- |
| 9.5. | Benzo (a)pyrene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.6. | Benzo (b)fluoranthene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.7. | Benzo (k)fluoranthene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.8. | Benzo[g,h,i]perylene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.9. | Chrysene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.10. | Dibenz[a'h]anthracene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.11. | Fluoranthrene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.12. | Fluorene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.13. | Indeno[123-c,]pyrene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.14. | Napthalene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.15. | Phenanthrene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| 9.16. | Pyrene | µg/L | BLQ (LOQ 0.01) | APHA 6440B 24 Ed. 2023 | Not Detectable |
| Pesticides Residue |  |  |  |  |  |
|  | Alpha –HCH | µg/L | BLQ (LOQ 0.01) | USEPA508 Rev 3.1: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Beta-HCH | µg/L | BLQ (LOQ 0.01) | USEPA508 Rev 3.1: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Gamma -HCH | µg/L | BLQ (LOQ 0.01) | USEPA508 Rev 3.1: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Delta – HCH | µg/L | BLQ (LOQ 0.01) | USEPA525.2: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Alpha- Endosulfan | µg/L | BLQ (LOQ 0.01) | USEPA508 Rev 3.1: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Beta endosulfan | µg/L | BLQ (LOQ 0.01) | USEPA525.2: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Endosulfan-sulfate | µg/L | BLQ (LOQ 0.01) | USEPA508 Rev 3.1: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Monocrotophos | µg/L | BLQ (LOQ 0.01) | USEPA8141 A: 1994 | 0.0001mg/L (0.1 µg/L) |
|  | Ethion | µg/L | BLQ (LOQ 0.01) | USEPA1657 A: 2000 | 0.0001mg/L (0.1 µg/L) |
|  | Chlorpyrifos | µg/L | BLQ (LOQ 0.01) | USEPA525.2: 1995 | 0.0001mg/L (0.1 µg/L) |

Continue…….

| Sample Name | Drinking Water | Test Report No. | FTL/AIPER/25/1812-A |
| --- | --- | --- | --- |
| Sample Details | 01 x 05 Litre | Registration No. | 713 |

| Sr. No. | Test Parameters | UoM | Result | Test Method | Specification |
| --- | --- | --- | --- | --- | --- |
|  | Phorate | µg/L | BLQ (LOQ 0.01) | USEPA8141 A: 1994 | 0.0001mg/L (0.1 µg/L) |
|  | Phorate sulfoxide | µg/L | BLQ (LOQ 0.01) | USEPA8141 A: 1994 | 0.0001mg/L (0.1 µg/L) |
|  | Phorate-sulfone | µg/L | BLQ (LOQ 0.01) | USEPA8141 : 1994 | 0.0001mg/L (0.1 µg/L) |
|  | Butachlor | µg/L | BLQ (LOQ 0.01) | USEPA525.2: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Isoproturon | µg/L | BLQ (LOQ 0.01) | USEPA525.2: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Alachlor | µg/L | BLQ (LOQ 0.01) | USEPA525.2: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Atrazine | µg/L | BLQ (LOQ 0.01) | USEPA525.2: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Parathion-methyl | µg/L | BLQ (LOQ 0.01) | USEPA8141: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Paraoxon- methyl | µg/L | BLQ (LOQ 0.01) | USEPA 8141 A: 1994 | 0.0001mg/L (0.1 µg/L) |
|  | Malathion | µg/L | BLQ (LOQ 0.01) | USEPA8141 : 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Malaoxon | µg/L | BLQ (LOQ 0.01) | USEPA8141: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Aldrin | µg/L | BLQ (LOQ 0.01) | USEPA525.2: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | Dieldrin | µg/L | BLQ (LOQ 0.01) | USEPA525.2: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | 2,4-DDT | µg/L | BLQ (LOQ 0.01) | USEPA508 Rev 3.1: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | 4,4-DDE | µg/L | BLQ (LOQ 0.01) | USEPA508 Rev 3.1: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | 2,4-DDD | µg/L | BLQ (LOQ 0.01) | USEPA508 Rev 3.1: 1995 | 0.0001mg/L (0.1 µg/L) |
|  | 4,4- DDD | µg/L | BLQ (LOQ 0.01) | USEPA 525.2 | 0.0001mg/L (0.1 µg/L) |
|  | 2,4- D | µg/L | BLQ (LOQ 0.05) | SOP 01 ISSUE NO. 1 | 0.0001mg/L (0.1 µg/L) |
|  | 2,4-DDE | µg/L | BLQ (LOQ 0.01) | USEPA508 Rev 3.1: 1995 | 0.0001mg/L (0.1 µg/L) |

Continue…….

| Sample Name | Drinking Water | Test Report No. | FTL/AIPER/25/1812-A |
| --- | --- | --- | --- |
| Sample Details | 01 x 05 Litre | Registration No. | 713 |

| Sr. No. | Test Parameters | UoM | Result | Test Method | Specification |
| --- | --- | --- | --- | --- | --- |
| Microbiological Test Parameters |  |  |  |  |  |
|  | E .coli | MPN/100ml | 58 | IS 1622:1981 | Should be absent |
|  | Total Coliform | MPN/100ml. | 63 | IS 1622:1981 | Should be absent |

Abbreviations used: UOM: Unit of Measurement; NMT: Not More Than; DOE: Date of Expiry; DOM: Date of Manufacturing;

NA: Not applicable, BLQ :Below Limit of Quantification , LOQ :Limit of Quantification A:Amendment

NOTE: 1) Report shall not be reproduced except in full without approval of the laboratory.

2) The results relate only to the items sampled / tested as received.

3) Duplicate report will be issued on chargeable basis.

4 This test report is the replacement to earlier test report no.. (FTL/AIPER/25/1812 ,Date 28/04/2025) earlier test                       report stands obsolete.

Reviewed by                                            Authorised Signatory                                                               Authorised Signatory

Mr. Vishal Deshmukh                              Ms. Monika Pali                                                                        Ms. Jyoti Pathak

Instrument Analyst                                  Technical Manager                                                                    Microbiology Head

*End of report*